The paper serves to detail the hypothetical development of an MSDAP (Mini Stereo Digital Audio Processor) that has the same capabilities as two general DSP chips used to implement two-channel FIR (finite impulse response) digital filtering in audio applications.

The algorithm for FIR digital filtering is comprised of the following linear convolution:

yn=k=0Nhk×xn-k

Where x(n) and y(n) are input and output audio sequences, and h(k) are filter coefficients with filter order N. The MSDAP will only perform the linear convolutions of y(n), so a simpler version of the general DSP chip hardware will be employed.

Starting with a general DSP as a reference, the first way to reduce the complexity of a FIR filter is to use power-of-two (POT) digits to approximate multiplication by approximating the coefficients using the nearest power-of-two digit as the coefficients instead of using a general multiplier with the same precision. The power-of-two utilizes bit shifters to achieve data values that correspond to the power-of-two approximation. In high-precision digital audio filters, the filter coefficients might need a big POT digit such as ±2-16. A programmable shifter from one to sixteen bits is not effective in hardware implementation as it takes up a lot of die area and a large critical path.

The issues of large bit-shifters have been mitigated by combining large bit-shifts into a series of addition/subtraction operations that can be expressed as:

yn=2-1…2-12-12-1u1+u2+ u3+…+u16)

and

u=xj1+xj2+…+xjrj

where 1 <= j <= 16.

This allows for the use of one-bit shifters within the MSDAP design. With these algorithms and FIR filtering structures implemented, the MSDAP will be able to implement two FIR filters with order up to N=255, and the total number R of addition/subtraction operation involved in computing the example the above equations is no more than 512.

The MSDAP will have to store these data samples in RAM. Virtual address can be created based on the filter POT coefficients when they are available and should have two bits attached to the address that indicate negative or positive coefficients and how many one-bit shifts are to be performed, with a total capacity of 2×156×16 bits (two channel audio signals are sampled with 16-bit precision). Only 1.8kBytes of RAM are required.

The clock rate of the system is dependent on which sample rate is wanted. In this case we have the highest sampling rate, 48kHz, which will require an MSDAP clock of 24.58 MHz. This can be changed to allow for lower power consumption at the expensive of performance.

Lastly, the implementation of circuit design needs to be considered. For certain circuit structures, such as introducing pipeline stage accumulators in ripple-carry adders would reduce the critical path time. NMOS only transmission gates are implemented to avoid the need for complementary clocks. A small Rom will be implemented containing a bootstrap program.  With these implementations in place, the MSDAP will have an active area of about 8mm2 in 1.0-u CMOS technology and its power dissipation is less than 100 mW from a 5-V supply.