SPRING25

Design Specification HW3

CE6306 ASIC Design HW3

Lamin Jammeh (DAL852207) and Caleb Perez (CXP180012)

Design Specification:

The MSDAP uses an ADD and Shift method to perform convolution on the input data_in Samples.  The filter in this case is a Finite Impulse Response (FIR). The Data_in samples are filtered using a filter coefficient and a filter order. Each input sample will go through all the filter orders represented by Rj and at each filter stage the input will be convoluted using the coefficients available at that  Rj stage.

Filter stage/ filter Tap: there are 16 filter stages Rj (where 0<j<16). Each filter stage is assign a 9-bit number, the MSB is the sign bit were 0= Positive sign bit (positive of the input signal will be used) and 1 = Negative sign bit (2,s complement of the input signal will be used). The table below shows and example of how the filter output is determined when n=3.

Pin Settings for the MSDAP chip:

The MSDAP chip will consist of pins. The Data_in pin as an input pin, the Data_in pin will feed into a temporary register of [1 x n x 40bit] called the Data_in Reg. Where n is number of possible samples the that can be process by the MSDAP. The last slot in the Data_in Reg is connected an output pin to let the Data_in pin knows the register is full and not to send any more Data. There will be a Reset (rst) pin which can be used clear both the Data_in Reg and Data_out Reg. the Signal clock (SCLK) pin will be used to send the clock signal. The SCLK enable data to move through the chip in a control manna. The output will have a Data_out pin that can handle a 40 bit out data. below is the chip outline.

|  |  | Coeff_Reg |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Rj_Reg |  |  |  |  |  |
|  | Data_in Reg |  |  |  | Data_out Reg |  |  |
| Data_in 40bit |  |  |  |  |  |  |  |
|  | x(n) |  |  |  | y(n) |  |  |
| Input_Busy 1bit | … |  |  |  | … | Data_out 40bit |  |
|  | … |  | 48KHz MSDAP CHIP |  | … |  |  |
| RST 1bit | … |  |  |  | … |  |  |
|  | x(1) |  | Adder |  | y(1) |  |  |
| SCLK | x(0) |  | Shifter |  | y(0) |  |  |

FSM Description:

A more FSM machine will be use to make there is no computation done between the different states of the state machine. The will be 16 case or state in the Moore FSM. Each state will be represented by Rj. Depending on the Rj value the corresponding coeeficients will be use to determine which memory location in the Data_in Reg the input sample will come from. After all the states in the FSM are completed or reset becomes high, new sets of input data will be stored in Data_in Reg.

SCLK:

For the Signal clock. The total number of coefficient of the filter was taking into account. Each Data_in sample will be filtered down to the last filter coffecient using addition/subtraction and then right shifting by 1 bit this makes the total computation at each filter coefficient to be 2. Choosing a sample frequency of 48KHz for the MSDAP.  The equations below are used to determine the Signal Clock.

Total cycle= Totalcoeff*Computation@stage-----Total cycles for each datainsample

Sampling Period= 1Selected Frequency-----freq=48KHz for our Design

clock period= Sampleing Periodtotal cycle  -----shortest cycle time

For our MSDAP the Coefficient register can hold 256 samples. Each location the coefficient register will carry to computations (add/subtract and shift). This gives the MSDAP 512 cycles. The table below shows how the SCLK frequency was determine using the above equations.

| System clock |  |  |
| --- | --- | --- |
| Number of Coeff | 256 | # of coifs |
| Number of computation at each stage | 2 | add + shift |
| total cycles for each data | 512 | cycles |
| Sampling Rate | 48 | KHz |
| Sampling Period @48KHz | 0.02083 | 20.83us |
| Data_in processing time | 4.0690E-05 | 40.7ns |
| SCLK Frequency | 24576 | 24.58KHz |