Design an ALU controller with the following specs

Inputs to the ALU:

- Sclk

- Start

- Reset_n

- ALU_FSM_en

- Rj

  - (rj is coming from a memory register of 16 rows long and 8bits wide)

- Coeff

  - (coeff is coming from a memory register of 256 rows long and 9bits wide)

- Data_in

  - (Data_in is coming from a memory register of 1000 column long and 16bits wide)

Ouputs of the ALU

- Rj_addr

  - (tells the rj_memory which address to send Rj from)

- Coeff_addr

  - (tells the coeff_memory which address to send Coeff from)

- Data_in_addr

  - (tells the data_in_memory which address to send Data_in from)

- Data_out

  - (once the data_in convolution is done the value is send to the Data_out)

- Read_rj_en

  - (enables the rj_memory to send rj to the ALU)

- Read_coeff_en

  - (enables the coeff_memory to send coeff to the ALU)

- Read_data_in_en

  - (enables the data_in_memory to send data_in to the ALU)

Temp register inside the ALU to keep the data during unfolding the loops

- Current_rj

  - (stores the current rj value)

- Current_coeff

  - (stores the current coeff value)

- Current_data_in

  - (stores the current data_in value)

- current_data_in_comp

  - (takes the 2s complement of the current data_in value and store)

- h

  - (this will store on the first 8 bits of the coeff example h<= current_coeff[7:0]

integers for tracking the loops

- interger n

  - (n will determine how many times the loop is iterated. N is depending on the total number of addresses in data_in_memory. Example if data_in[0 to 999] therefore n[0 to 999], and we have data_out[0] to data_out[999]. In essence it will be the outer loop)

- integer rj_counter

  - (this will be the second outer layer of the loop since each data_out[n] will go through all the rj values. The rj_counter will keep track of the rj_addr. Since the rj_addr will go from 0 to 15. This counter only clears or zeros on reset. Rj will determine how many coeff values will be use at each loop.

    - Example rj_counter=0 then rj_addr=0000

    - If the value of rj at rj[0]=1011 then the first coeff_addr=0 to coeff_addr=10 will be activated

  - Note in the next loop rj_counter will be incremented by 1but n loop will stay the same until all of the rj_addr are complete)

- interger coeff_counter

  - (this is the inner loop of the n iteration. this will keep track of the last coeff_addr that was used. It will also clear or zeros on reset. It will be use to determine which coeff_addr are going to be use in that loop.

  - All the sets of coeff_addr for the particular current rj_value is executed before the inner loop moves out to the middle loop

  - The coeff_counter is accumulated at the begine of the inner loop

    - Example if the current coeff_counter= 5 and the next rj value is 7

    - Then the coeff_counter=12 but only coeff_addr 5 to 11 will be used at that stage

- interger k

  - ( k is going to be the current value in h. example if h=00000101 then k=5. Note k will be used with n to determine the index)

- interger index

  - (index will be used to determine which data_in_addr will be used at that particular filter coeff. Therefore index=(n-k), since there will be no negative address if index<0 then data_in will be 0. Note is how it will work data_in[data_in_addr] were data_in_addr=index)

Determine data_out

Once the index is determined, and data_in is obtain the following will happen

Data_out <= Current_data_in if MSB of the current_coeff is 0

Otherwise Data_out <= Current_data_in_comp

In the future will be used make another block that accumulates the data_out as SUM block